Multimode Robotic Car Projects - Complete Documentation

1. Detailed Project Catalog

WiFi Controlled Car

• Modes Supported: Remote / Autonomous / Web UI

• Mode Explanations: Remote: app control; Autonomous: sensor-based driving; Web UI: browser control

• Key Features: ESP32 WiFi, Cloudflare relay, browser UI

• Components/Materials: ESP32, motor driver, WiFi module, IR sensors, OLED

• Sensors (Number & Type): 2 IR, 1 ultrasonic

• Best Use Case: IoT demonstrations

Bluetooth Rover

• Modes Supported: Manual / Bluetooth / Autonomous

• Mode Explanations: Manual: joystick; Bluetooth: mobile app; Autonomous: sensor navigation

• Key Features: HC-05 module, mobile app integration

• Components/Materials: Arduino UNO, HC-05, motor driver, IR sensors

• Sensors (Number & Type): 2 IR

• Best Use Case: Classroom workshops

Gesture-Controlled Car

• Modes Supported: Gesture / Manual / Autonomous

• Mode Explanations: Gesture: accelerometer input; Manual: joystick; Autonomous: IR sensors

• Key Features: Accelerometer-based gesture input

• Components/Materials: Arduino, accelerometer sensor, motor driver, IR sensors

• Sensors (Number & Type): 1 accelerometer, 2 IR

• Best Use Case: Robotics competitions

Line Follower Car

• Modes Supported: Line-follow / Manual / Obstacle

• Mode Explanations: Line-follow: IR sensors detect path; Manual: joystick; Obstacle avoidance: ultrasonic

• Key Features: IR sensors, PID tuning

• Components/Materials: Arduino UNO, IR sensors, ultrasonic sensor, motor driver

• Sensors (Number & Type): 3 IR, 1 ultrasonic

• Best Use Case: STEM education

Obstacle Avoidance Car

• Modes Supported: Autonomous / Manual / Remote

• Mode Explanations: Autonomous: ultrasonic sensors; Manual: joystick; Remote: app control

• Key Features: Ultrasonic + IR sensors, collision prevention

• Components/Materials: Arduino UNO, ultrasonic sensor, IR sensors, motor driver

• Sensors (Number & Type): 2 IR, 2 ultrasonic

• Best Use Case: Beginner robotics projects

Voice-Controlled Car

• Modes Supported: Voice / Manual / Remote

• Mode Explanations: Voice: speech recognition app; Manual: joystick; Remote: mobile app

• Key Features: Speech recognition via app

• Components/Materials: Arduino UNO, microphone module, Bluetooth/WiFi, motor driver

• Sensors (Number & Type): 1 microphone, 2 IR

• Best Use Case: Interactive demos

GPS Navigation Car

• Modes Supported: GPS / Remote / Autonomous

• Mode Explanations: GPS: outdoor navigation; Remote: app control; Autonomous: sensor-based

• Key Features: GPS module, outdoor navigation

• Components/Materials: Arduino/ESP32, GPS module, motor driver, IR sensors

• Sensors (Number & Type): 1 GPS, 2 IR

• Best Use Case: Smart city projects

Solar-Powered Rover

• Modes Supported: Solar / Battery / Remote

• Mode Explanations: Solar: solar panel; Battery: rechargeable pack; Remote: app control

• Key Features: Dual power system, eco-friendly design

• Components/Materials: Arduino UNO, solar panel, battery pack, motor driver

• Sensors (Number & Type): 2 IR

• Best Use Case: Sustainability education

Camera Vision Car

• Modes Supported: Camera / Autonomous / Remote

• Mode Explanations: Camera: computer vision; Autonomous: ML integration; Remote: app control

• Key Features: Computer vision, ML integration

• Components/Materials: ESP32-CAM, motor driver, IR sensors, WiFi module

• Sensors (Number & Type): 1 camera, 2 IR

• Best Use Case: AI workshops

Omni-Wheel Car

• Modes Supported: Manual / Autonomous / Omni-directional

• Mode Explanations: Manual: joystick; Autonomous: sensor-based; Omni-directional: 360° wheel movement

• Key Features: 360° movement, advanced motor driver

• Components/Materials: Arduino/ESP32, omni-wheels, motor driver, IR sensors

• Sensors (Number & Type): 4 IR

• Best Use Case: Advanced robotics labs

2. DIY Materials List

WiFi Controlled Car

2WD chassis, 2 DC motors, ESP32, L298N motor driver, IR sensors (x2), Ultrasonic sensor, OLED display, WiFi module (built-in ESP32), Jumper wires, Battery pack (7.4V Li-ion), Breadboard, Screws & nuts, Wheels (x2), Caster wheel

Bluetooth Rover

2WD chassis, 2 DC motors, Arduino UNO, HC-05 Bluetooth module, L298N motor driver, IR sensors (x2), Jumper wires, Battery pack (9V or 7.4V Li-ion), Breadboard, Wheels (x2), Caster wheel, Screws & nuts

Gesture-Controlled Car

2WD chassis, 2 DC motors, Arduino UNO, Accelerometer (e.g., ADXL335), L298N motor driver, IR sensors (x2), Jumper wires, Battery pack, Breadboard, Wheels (x2), Caster wheel

Line Follower Car

2WD chassis, 2 DC motors, Arduino UNO, IR sensors (x3), Ultrasonic sensor, L298N motor driver, Jumper wires, Battery pack, Breadboard, Wheels (x2), Caster wheel

Obstacle Avoidance Car

2WD chassis, 2 DC motors, Arduino UNO, Ultrasonic sensors (x2), IR sensors (x2), L298N motor driver, Jumper wires, Battery pack, Breadboard, Wheels (x2), Caster wheel

Voice-Controlled Car

2WD chassis, 2 DC motors, Arduino UNO, Microphone module, Bluetooth module (HC-05/HC-06), IR sensors (x2), L298N motor driver, Jumper wires, Battery pack, Breadboard, Wheels (x2), Caster wheel

GPS Navigation Car

2WD chassis, 2 DC motors, Arduino UNO or ESP32, GPS module (e.g., NEO-6M), IR sensors (x2), L298N motor driver, Jumper wires, Battery pack, Breadboard, Wheels (x2), Caster wheel

Solar-Powered Rover

2WD chassis, 2 DC motors, Arduino UNO, Solar panel, Rechargeable battery pack, IR sensors (x2), L298N motor driver, Jumper wires, Breadboard, Wheels (x2), Caster wheel

Camera Vision Car

2WD chassis, 2 DC motors, ESP32-CAM, IR sensors (x2), L298N motor driver, WiFi module (built-in), Jumper wires, Battery pack, Breadboard, Wheels (x2), Caster wheel

Omni-Wheel Car

Omni-directional chassis, 4 DC motors, Arduino UNO or ESP32, IR sensors (x4), Advanced motor driver (e.g., TB6612FNG), Jumper wires, Battery pack, Breadboard, Omni wheels (x4), Screws & nuts

3. Technical Build Manual

WiFi Controlled Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Bluetooth Rover

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Gesture-Controlled Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Line Follower Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Obstacle Avoidance Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Voice-Controlled Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

GPS Navigation Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Solar-Powered Rover

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Camera Vision Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

Omni-Wheel Car

Circuit Diagram Description:

Sensors send signals to the microcontroller, which processes the data and controls the motor driver to operate the motors.

Pin Mapping Table (Example):

| Component | Pin |
| --- | --- |
| IR Sensor 1 | D2 |
| IR Sensor 2 | D4 |
| Ultrasonic Trigger | D5 |
| Ultrasonic Echo | D6 |
| Motor Driver IN1 | D7 |
| Motor Driver IN2 | D8 |
| Motor Driver IN3 | D9 |
| Motor Driver IN4 | D10 |

Troubleshooting Tips:

• Check wiring connections and power supply.
• Verify sensor orientation and functionality.
• Ensure code is uploaded correctly to the microcontroller.

4. Workshop Guide

This section provides a simplified overview for students during workshops.

WiFi Controlled Car

Modes: Remote / Autonomous / Web UI

Key Components: ESP32, motor driver, WiFi module, IR sensors, OLED

Use Case: IoT demonstrations

Bluetooth Rover

Modes: Manual / Bluetooth / Autonomous

Key Components: Arduino UNO, HC-05, motor driver, IR sensors

Use Case: Classroom workshops

Gesture-Controlled Car

Modes: Gesture / Manual / Autonomous

Key Components: Arduino, accelerometer sensor, motor driver, IR sensors

Use Case: Robotics competitions

Line Follower Car

Modes: Line-follow / Manual / Obstacle

Key Components: Arduino UNO, IR sensors, ultrasonic sensor, motor driver

Use Case: STEM education

Obstacle Avoidance Car

Modes: Autonomous / Manual / Remote

Key Components: Arduino UNO, ultrasonic sensor, IR sensors, motor driver

Use Case: Beginner robotics projects

Voice-Controlled Car

Modes: Voice / Manual / Remote

Key Components: Arduino UNO, microphone module, Bluetooth/WiFi, motor driver

Use Case: Interactive demos

GPS Navigation Car

Modes: GPS / Remote / Autonomous

Key Components: Arduino/ESP32, GPS module, motor driver, IR sensors

Use Case: Smart city projects

Solar-Powered Rover

Modes: Solar / Battery / Remote

Key Components: Arduino UNO, solar panel, battery pack, motor driver

Use Case: Sustainability education

Camera Vision Car

Modes: Camera / Autonomous / Remote

Key Components: ESP32-CAM, motor driver, IR sensors, WiFi module

Use Case: AI workshops

Omni-Wheel Car

Modes: Manual / Autonomous / Omni-directional

Key Components: Arduino/ESP32, omni-wheels, motor driver, IR sensors

Use Case: Advanced robotics labs

5. Client Proposal

Executive Summary:

This document presents a comprehensive suite of multimode robotic car projects designed for education, innovation, and demonstration purposes. Each project integrates modern technologies such as IoT, AI, GPS, and renewable energy.

Benefits:

• Enhances STEM education
• Promotes hands-on learning
• Demonstrates real-world applications of robotics
• Scalable for classrooms, workshops, and exhibitions

Scalability:

Projects can be scaled from individual DIY kits to full classroom sets. Each project is modular and can be customized based on budget and learning objectives.

6. Cost & Sourcing Estimates

Estimated costs for DIY materials (in NPR and USD):

| Project | Estimated Cost (NPR) | Estimated Cost (USD) | Notes |
| --- | --- | --- | --- |
| WiFi Controlled Car | 3500 | 26 | Includes all listed components |
| Bluetooth Rover | 3500 | 26 | Includes all listed components |
| Gesture-Controlled Car | 3500 | 26 | Includes all listed components |
| Line Follower Car | 3500 | 26 | Includes all listed components |
| Obstacle Avoidance Car | 3500 | 26 | Includes all listed components |
| Voice-Controlled Car | 3500 | 26 | Includes all listed components |
| GPS Navigation Car | 3500 | 26 | Includes all listed components |
| Solar-Powered Rover | 3500 | 26 | Includes all listed components |
| Camera Vision Car | 3500 | 26 | Includes all listed components |
| Omni-Wheel Car | 3500 | 26 | Includes all listed components |

7. Student Workbook

Practice Questions:

1. Which car uses GPS for navigation?

2. Name two projects that support autonomous mode.

3. Which sensors are used in the Line Follower Car?

4. What is the best use case for the Solar-Powered Rover?

5. Match the following components to their function.

- IR Sensor → ______
   - Motor Driver → ______
   - ESP32 → ______

Notes Space: