# Part II: DIY Electronics Projects for Students

# Chapter 1: MIY (Make It Yourself) Touch Sensor

Objective: To demonstrate how a transistor can be used as a touch-activated switch to control an AC bulb.

## Materials Required

🔋 Transistor (BC547)

🧵 Jumper wires

🔋 Battery (9V)

⚡ Relay

💡 Bulb (AC)

🔌 AC & DC Power source

## Step-by-Step Build

Connect battery to transistor base via jumper wires.

Wire transistor collector to relay input.

Connect relay output to bulb and AC source.

Ground the transistor emitter.

Test by touching wires — bulb should glow.

## Working Principle

Transistor conducts when base voltage > 0.7V.

Touching wires completes circuit → base current flows.

Relay energizes when transistor conducts.

Relay closes AC circuit → bulb ON.

Removing touch → transistor OFF → relay OFF → bulb OFF.

## Observation

When the wires are touched, the bulb lights up, indicating the transistor is conducting and the relay is activated.

# Chapter 2: Street Light Automation System

Objective: To automate street lighting based on ambient light using an LDR and Arduino.

## Materials Required

🖥️ Microprocessor (Arduino UNO)

🌞 LDR

⚡ Relay module

💡 Bulb

🔌 AC & DC Power source

## Step-by-Step Build

Connect LDR to Arduino analog input.

Program Arduino to read resistance.

Connect relay module to Arduino output.

Wire relay to bulb and AC source.

Upload code and test.

## Working Principle

LDR resistance high in dark, low in light.

Arduino compares resistance to threshold.

Dark → relay ON → bulb ON.

Light → relay OFF → bulb OFF.

## Observation

The bulb turns ON in darkness and OFF in light, confirming the LDR and Arduino logic works correctly.

# Chapter 3: Smart Farm System

Objective: To automate irrigation and provide a security alarm for farms using sensors and Arduino.

## Materials Required

🖥️ Arduino UNO

🌱 Soil moisture sensor

⚡ Relay module

💧 Water pump

🔌 AC & DC Power source

🔋 Transistor (BC547)

🔔 Buzzer

🔋 Battery (9V)

🧵 Jumper wires

## Step-by-Step Build

Smart Irrigation:

Connect soil sensor to Arduino input.

Connect relay to Arduino output.

Wire relay to pump + AC source.

Upload code to activate pump when soil dry.

Smart Alarm Fence:

Connect wires to transistor base.

Connect transistor collector to buzzer.

Power with 9V battery.

Test by touching wires.

## Working Principle

Soil sensor outputs ‘0’ when dry, ‘1’ when wet.

Arduino turns relay ON if input = ‘0’ → pump ON.

Arduino keeps relay OFF if input = ‘1’ → pump OFF.

Fence wires touched → transistor base current flows → buzzer ON.

Wires untouched → transistor OFF → buzzer OFF.

## Observation

Pump activates when soil is dry and buzzer sounds when fence wires are touched, confirming both systems function as intended.

# Chapter 4: Smart Tap System

Objective: To create a touchless water dispensing system using a PIR sensor and Arduino.

## Materials Required

🖥️ Microprocessor (Arduino UNO)

👋 PIR sensor

⚡ Relay module

💧 Water pump

🔌 AC & DC Power source

## Step-by-Step Build

Connect PIR sensor to Arduino input.

Connect relay to Arduino output.

Wire relay to pump + AC source.

Upload code to activate pump when PIR detects presence.

## Working Principle

PIR detects infrared radiation from nearby object.

Sensor sends ‘1’ when object detected.

Arduino turns relay ON → pump ON.

Sensor sends ‘0’ when no object → relay OFF → pump OFF.

## Observation

Water pump activates when motion is detected, confirming the PIR sensor and relay control work properly.

# Chapter 5: Smart Door Lock System and Weather Station

Objective: To build a secure door lock system and a weather monitoring station using Arduino.

## Materials Required

🖥️ Arduino UNO

⌨️ Keypad

📶 RFID module

📺 LCD

🔧 Servo motor

🔌 AC power supply

🌡️ DHT11 sensor

## Step-by-Step Build

Door Lock:

Connect keypad and RFID module to Arduino inputs.

Connect servo motor to Arduino output.

Connect LCD to Arduino for display.

Program Arduino to validate keypad/RFID inputs.

Servo rotates 90° if input is valid, then returns after 3 seconds.

Weather Station:

Connect DHT11 sensor to Arduino input.

Connect LCD to Arduino output.

Program Arduino to read sensor values and display them.

## Working Principle

Keypad/RFID sends input data to Arduino.

Arduino compares input with stored values.

If valid → servo rotates 90° → door opens.

After 3s → servo returns → door closes.

DHT11 measures temperature and humidity.

Arduino reads sensor → displays values on LCD.

## Observation

Door opens only with valid input and weather data is displayed on LCD, confirming both systems operate correctly.

# Chapter 6: Smart Home System

Objective: To automate lighting and door control using multiple sensors and Arduino.

## Materials Required

🖥️ Arduino UNO & NodeMCU

📏 Ultrasonic sensor

🌞 LDR

⚡ Relay module

💡 Bulb

👆 Touch sensor

🔧 Servo motor

🧵 Jumper wires

🔌 DC & AC power supply

## Step-by-Step Build

Smart Compound Light:

Connect LDR and ultrasonic sensor to Arduino inputs.

Connect relay module to Arduino output.

Wire relay to bulb and AC source.

Program Arduino to check night condition + person presence.

Smart Main Door:

Connect touch sensor to Arduino input.

Connect servo motor to Arduino output.

Program Arduino to rotate servo 90° when sensor touched.

Servo returns after 3s.

## Working Principle

LDR detects darkness (high resistance).

Ultrasonic detects person within 20 cm.

Arduino checks both conditions → relay ON → bulb ON.

Daytime or no person → relay OFF → bulb OFF.

Touch sensor sends ‘1’ when pressed.

Arduino rotates servo 90° → door opens.

After 3s → servo returns → door closes.

## Observation

Bulb lights up only at night with presence detected and door opens on touch, confirming smart automation works as intended.